INTEGRATED WASTE MANAGEMENT PLAN (IWMP)

29 May 2019

Dr Nkosazana Dlamini Zuma Local Municipality

CONTENTS

1.	Defining the geographical area	4

2.	Situation analysis	5

2.1.	Demographics (Population and development profiles)	5

2.1.1.	Growth&Demographic Profile	5

2.1.2.       Dwelling Types...........................................................................................................................7

2.2.	Determining current waste generation and estimating future waste generation rates and quantities	9

2.2.1.	Domestric Waste Generation	9

2.3.	Waste quantities and types	13

2.3.1.	Weighbridge	13

2.3.2.	Volume density estimation system	14

2.3.3.	Waste stream analysis	15

2.4.	Waste recycling, treatment and disposal	16

2.4.1.	Status Quo of Waste Disposal Facilities	16

2.4.2.	Status Quo of Waste Treatment Facilities	17

2.4.3.	Status Quo of Waste Recyclers	18

2.4.4.	Status Quo of other types of facilities	19

2.5.	Status of waste collection services	20

2.5.1.	National Domestic Waste Collection Standandards.................................................20

2.6.	Financing of Waste Management	24

2.6.1.	Budget/Expenditure: Income and expenditure	24

2.6.2.	Revenue sources	26

2.6.4.	Organisational and institutional matters	27

3.	Desired end state	28

3.1.	Setting strategic goals, targets and indicators	28

4.	Identify, evaluate and select alternatives	31

4.1.	Strategic goals, targets, timeframe, budget	31

5.	Communication and Stakeholder Participation	34

5.1.	Consultation Process Summary	34

6.	Implementation Instruments	35

6.1.	Partnerships	35

6.2.	Legislative instruments: Development and enforcement of by-laws	36

6.3.	Funding mechanisms	37

6.4.	Implementation Plan (Summary of an IWMP Planning Process)	38

7.	Reporting on Monitoring	40

7.1.	Strategic issues	41

7.2.	Performance	42

7.3.	Public accountability	43

7.4.	Communication and public participation plan	44

7.5.	Financial plan	45

7.6.	Waste management implementation programme	46

Annexure or References (Optional)	47

## Defining the geographical area

Dr Nkosazana Dlamini-Zuma Local Municipality is a local municipality established after the August 2016 local government elections. It followed a gazetted merger between erstwhile Ingwe and Kwa Sani local municipalities. The new NDZ Local Municipality (LM) is situated in the southern part of KwaZulu-Natal. It is located within Harry Gwala District Municipality (HGDM), the fourth largest district municipality in Kwa-Zulu Natal. The municipality is a predominantly rural municipality with 5 main towns within its boundaries. These include Underberg, Himeville, Creighton, Bulwer, and Donnybrook. It is a category B municipality and is deemed the second largest municipality in the District in terms of population size.  The Local municipalities within Harry Gwala District include:Dr Nkosazana Dlamini Zuma Local Municipality; Umzimkhulu LM; Ubuhlebezwe LM and Greater Kokstad LM (Draft IDP, 2019/20).

1.5.	LAND COVER AND BROAD LAND USES

1.5.1.	TOPOGRAPHY

The altitude ranges from 2083 metres above sea level in the north east (aMahwaqa Peak) to a low of approximately 450 metres at the bottom of the Umkhomazi River valley in the south.

Dr Nkosazana Dlamini-Zuma Local Municipality comprises of gently undulating to steeply undulating land.  Much of the gently sloped land is restricted to small “plateaus”, which are primarily found in the western highlands areas.

1.5.2.	GEOLOGY

The geological nature of an area influences the topography, and alignment of river channels. It also has an influence on the type of soil formations prevalent. The municipal area is underlined by rock derived from dolerite and mudstones.  The eastern lower lying areas of the municipality are dominated by shale’s and arsenate.  The soils are generally considered to have low fertility.

The second very important common characteristic evident in most of the soils within the municipal area is that they are highly erodible. Majority of the municipal area (northwestern portion of the municipality moving towards the central portion) is characterized by mudstone and dispersed with dolerite. The southern and southeastern portion of the municipality is dominated by shale, dispersed with Ecca Group Arenite.

1.8.1.	VEGETATION

Dr Nkosazana Dlamini-Zuma Local Municipality has a wide diversity of vegetation.  This includes vegetation that is well represented elsewhere in the province, vegetation that is of particular ecological interest (such as the plant communities that are associated with the dolerite dykes in the area).

The variety of vegetation types is widespread throughout the NDZ municipal area. Safe to highlight that there are two main vegetation types, which have been identified as being the most significant to the biodiversity in the area. They are namely: -

Mist belt grassland- This vegetation type is endemic to KZN and has a high biodiversity value due to the lack of representation of true Mist belt grassland inside and outside the formally protected areas. The Mist belt grassland plays an important role in the provision of a suitable habitat for endangered blue swallow and Oribi.

Mist belt Forests- This forest is probably the most important forest in the area. This is mainly because it is one of the few forests with a high number of Cape Parrots and it has the Tree Hyrax. Therefore, the conservation of this forest is of great importance.

The vegetation in Dr Nkosazana Dlamini-Zuma Local Municipality contains several environmentally important and sensitive vegetation types.  The area can be divided into seven Bio-resource groups namely,

- Moist Highveld Sourveld (24%),

- Dry Highveld Sourveld (<1%),

- Moist Transitional Tall Grassveld (60%)

- Moist Midlands Mist belt (4%),

- Moist Tall Grassveld (6%),

- Coast Hinterland Thornveld (<1%), and

- Valley Bushveld (5%)

Aside from the indigenous (natural) vegetation, there are extensive areas upon which agricultural activities have affected significant changes to the vegetation.  Amongst the most obvious of these are the commercial forestry operations.

Dr Nkosazana Dlamini-Zuma Local Municipality has an important role to play in ensuring that both conservation and development are complimentary to one another and ensure that the natural environment is not compromised. It is recommended that development procedures be carried through thoroughly and concisely for all developments taking place within the municipality, more particularly around natural water resources and wetland features.

1.8.2.	AGRICULTURE

1.8.2.1.	METHODS APPLIED IN ASSESSING AGRICULTURAL POTENTIAL

This information is drawn from the Harry Gwala District Rural Plan undertaken in 2015. It highlights that several methods for determining agricultural potential at a high level without having visited an area exist and all these methods rely on Global Information System (GIS) data that have been gathered and interpreted by National Government Departments.  Datasets referred to during this study are described briefly as follows:

	Land capability – developed by the Agricultural Research Council, land capability GIS data is used to classify land into 8 distinct classes that vary by land-use which can either be arable, grazing and wildlife;

	Bioresource Units (BRUs) –Bioresource Units GIS data is available from the KZN Department of Agriculture and Rural Development (KZNDARD) and uses GIS data to classify units of land where a defined area in which the environmental and physical conditions such as soil types, vegetation, climate and terrain form are sufficiently uniform can be identified and a land management practice recommended; and

	Agricultural Land Categories – the Agricultural Land Categories GIS data compiled by the Natural Resources division of the KZN DARD is by the most recent and comprehensive dataset available and combines various planning datasets to produce a composite layer that can be used for fairly detailed development planning.

Each of the abovementioned methods and datasets are discussed in greater detail in the following sections.

1.9.	ENVIRONMENTAL ANALYSIS

1.9.1.	BIODIVERSITY

The effective conservation of the world’s biodiversity results in the long-term survival and well-being of the people. Pressures on biodiversity show no sign of decreasing, yet resources for conservation action are limited. Dr Nkosazana Dlamini- Zuma  Local Municipalilty needs to be strategic and focus efforts where they will have the greatest impact.  Mindset is employed as a data analysis function that identifies a “minimum set” of planning units that will assist in meeting conservation targets. Although no detailed sampling of the fauna of the entire Dr Nkosazana Dlamini- Zuma  Local Municipalilty municipal area has been completed, available data indicates that in terms of game animals, species diversity is quite low due to the dominance of Sourveld type grasslands (Dr Nkosazana Dlamini-Zuma Local Municipality Idp 2002).  However, there are a number of common, rare and endangered species present.

In terms of high biodiversity value, there are several areas in the municipality identified in terms of priority 1. This is due to the following animals, which are Red Data species found in the Dr Nkosazana Dlamini- Zuma Local Municipalilty. They have high biodiversity value; as such, they need protection from exploitation and habitat loss:

- Oribi: vulnerable but bordering on endangered

- Blue swallow: critically endangered

- Cape parrot: critically endangered

- Wattle crane: critically endangered

- Blue crane: endangered

- Crowned crane: endangered

- Cape vulture: endangered

- Tree hyrax: endangered

The Harry Gwala DM has developed a District Biodiversity Sector Plan. It takes extensive cognizance of the KZN Provincial Biodiversity Plan developed by KZN Wildlife. The District Biodiversity Plan identifies the environmentally sensitive areas, conservation and protected areas as well as the Critical Biodiversity Areas (CBAs). The CBAs are considered as areas critical to meeting biodiversity targets and thresholds. They are crucial to maintain viable population of species as well as the functionality of ecosystems (Escott, et al. 2013).

1.9.2.	KEY HYDROLOGICAL FEATURES

The municipal area covers an extremely diverse and unique environment which encompasses vast open grasslands, highly productive arable farmlands, pristine rivers with their huge network of tributaries, numerous wetlands, lakes and dams as well as towering mountains and the foothills leading into them.  These physical and scenic resources coupled with a rich diversity of Fauna and Flora as well as cultural and historical resources (e.g. San rock art) have earned the area the reputation of being a most important Inland Tourism Destination. At the same time the high rainfall and abundant water supply in the form of streams and rivers, good natural grazing and some high potential arable soils contribute to the area’s considerable agricultural output.

Emphasis will be placed on:

	Ensuring that productive agricultural areas are not lost to the region by ensuring that proposed new non-agricultural development is carefully controlled and only allowed to take place in the designated development nodes.

	Encouraging sound farming and land use practices especially amongst the subsistence and emerging farmers in the tribal areas.

	Education of the community in the importance and financial benefits of maintaining the environment in a healthy state:  The value of maintaining landscape quality cannot be recognized and where necessary reclaiming degraded landscapes due to erosion, overgrazing and other poor land management practices.  The need to control or remove alien and invasive plants is of importance here.

	Monitoring and controlling all activities within the Drakensberg Heritage Site Buffer Zone, even where they take place within the designated development nodes.  Here again the importance of protecting existing landscape quality is paramount.  The importance of preserving and protecting the natural environment is well recognized by the municipality as it will have an enormous influence on the future quality of life and financial well-being of the whole community.

	The preservation of the agricultural component, increase in tourism and general attractiveness for people to settle here depend on it.

1.9.3.	AIR QUALITY

Due to the rural nature of Dr Nkosazana Dlamini-Zuma Local Municipality, the air quality is considerably good. The dispersed housing and numerous plantations found in and around the area allow for a good oxygen carbon dioxide cycle. Thus, the only air pollution that could endanger the livelihoods of the inhabitants would be the burning of wood, forest waste and fugitive dust emissions generated from unpaved roads.

1.9.4.	CLIMATE AND CLIMATE CHANGE

Two bio-climatic regions exist.  These are the highland and the moist upland bio-climatic regions.  The area can be categorized into two temperature zones:

	The western (higher) portions of Dr Nkosazana Dlamini-Zuma Local Municipality have good climate and are typically cooler. Winter temperatures in the cooler western regions often drop below 00 C.

	The eastern (lower) portions can be described to have high climate. Warmer eastern regions temperatures seldom drop below 50 C.

Summer temperatures range from the low thirties in the west to high thirties in the east. The mean annual rainfall in the area is between 700 and 1200mm per annum with the eastern areas generally being drier than those in the west are.

1.9.4.1.	EXTREME WEATHER RISKS

There is variability in the features that influence the region’s climate. This produces extreme weather conditions in Dr Nkosazana Dlamini-Zuma Local Municipality. Some of the most common climate extremes cause serious impacts. These often record numerous deaths, damage to households (leaving people homeless), create health concerns and require efficient emergency assistance. It also affects negatively on biodiversity.

Map 1 Geographic Location of Dr Nkosazana Dlamini Zuma Local Municipality

## Situation analysis

### Demographics (Population and development profiles)

2.1.1     Growth & Demographic Profile

Dr Nkosazana Dlamini Zuma Local Municipality is composed of a total population of 118480 as indicated in table below (Stats SA, community survey 2016).  Dr Nkosazana Dlamini-Zuma Local Municipality is a predominantly rural municipality with five main towns within its boundaries; these include Creighton, Bulwer, Donnybrook, Underberg and Himeville (KwaSani IDP, 2015/16; KZN 436 IDP, 2015/16).

| Growth estimates |  | 2011 | 2016 |  |
| --- | --- | --- | --- | --- |
| Municipality Population (as per Census 2011): | 100548 |  |  | 118480 |
| Estimated Population Growth (%) as per Census 2011: | -0.67 |  |  | 1.67 |
| Estimated current population as of 2016 (year is a parameter): | 97179.6 |  |  |  |
| Demographic profile |  |  |  |  |
| Age: |  |  |  |  |
| Youth | 73858 |  |  | 92414 |
| Middle age | 18842 |  |  | 17973 |
| Old age | 7849 |  |  | 8092 |
| Gender: |  |  |  |  |
| Male | 46752 |  |  | 56732 |
| Female | 53795 |  |  | 61748 |
| Education: |  |  |  |  |
| Primary | 40766 |  |  |  |
| Secondary | 35189 |  |  |  |
| Tertiary | 2088 |  |  |  |
| Employment: |  |  |  |  |
| Employed | 10570 |  |  |  |
| Unemployed | 57342 |  |  |  |

Population distribution graphs:

#### 2.1.2 Dwelling Types

| House or brick/concrete block structure on a separate stand or yard or on a farm: | 5269 |
| --- | --- |
| Traditional dwelling/hut/structure made of traditional materials: | 15789 |
| Flat or apartment in a block of flats: | 917 |
| Cluster house in complex: | 97 |
| Townhouse (semi-detached house in a complex): | 66 |
| Semi-detached house: | 15 |
| House/flat/room in backyard: | 533 |
| Informal dwelling (shack in backyard): | 96 |
| Informal dwelling (shack not in backyard e.g. in a informal/squatter settlement or on a farm: | 67 |
| Room/flatlet on a property or larger dwelling/servants quarters/granny flat: | 40 |
| Caravan/tent: | 20 |
| Other: | 143 |

Dwelling types graphs:

### Determining current waste generation and estimating future waste generation rates and quantities

#### Domestic Waste generation

| Current waste generation and estimated future waste generation |  |
| --- | --- |
| Current domestic waste generation rates | 434.68 |
| Future domestic waste generation rates (in 10 years) | 4346.8 |

Domestic waste generation graph:

### Waste quantities and types

#### Weighbridge

Dr Nkosazana Dlamini-Zuma Municipality consist of two landfill sites which are Creighton and Bulwer landfill site and both of them do not have a weighbridge. The Himeville Transfer Station that receives large quantities of waste does not have a weighbridge. The waste is quantified based on estimate system.

#### Volume density estimation system

| Creighton Waste Disposal site |  |  |
| --- | --- | --- |
| Waste type/streams | Waste generated per annum (tons) | Total percentages |
| Organic waste | 0 | 0 % |
| Cans | 0 | 0 % |
| Paper | 0 | 0 % |
| Glass | 0 | 0 % |
| Plastic | 0 | 0 % |
| Construction and demolition waste | 0 | 0 % |
| Tyres | 0 | 0 % |
| Other | 0 | 0 % |
| Total |  | 100 % |

| Bulwer Waste Disposal Site |  |  |
| --- | --- | --- |
| Waste type/streams | Waste generated per annum (tons) | Total percentages |
| Organic waste | 0 | 0 % |
| Cans | 0 | 0 % |
| Paper | 0 | 0 % |
| Glass | 0 | 0 % |
| Plastic | 0 | 0 % |
| Construction and demolition waste | 0 | 0 % |
| Tyres | 0 | 0 % |
| Other | 0 | 0 % |
| Total |  | 100 % |

#### Waste stream analysis

The table below shows the estimated results that were obtained during Waste categorization exercise that was performed by Department of Environmental Affairs in partnership with Dr Nkosazana Dlamini Zuma Municipality on the 28 May 2019 on the Himville Transfer Station.

| Himeville Transfer Station |  |  |
| --- | --- | --- |
| Waste type/streams | Waste disposed per annum (tons) | Total percentages |
| Organic waste | 11 | 17.7419 % |
| Cans | 8 | 12.9032 % |
| Paper | 6 | 9.67742 % |
| Glass | 16 | 25.8065 % |
| Plastic | 21 | 33.871 % |
| Construction and demolition waste | 0 | 0 % |
| Tyres | 0 | 0 % |
| Other | 0 | 0 % |
| Total |  | 100 % |

The residential waste stream shows that there is various types of waste within Himville Transfer Station which includes cardbox, clear plastic, glass, paper, textile, PET, polysterene, organic, garden, MCW, metal, nappies, HDPE, ewaste and hazardous waste.

The hotel waste stream is dominated by paper, organic and nappies waste. The availability of other waste types including metal, glass, textile, clear plastic, MCW, garden, polystyrene, cardbox, other plastic, HDPE and textile was also observed.

Soldier Stream

The Soldier camp waste stream categorization shows that there is a lot of waste that comes from this camp. The presence of all waste types was observed.

| Waste Stream | Sample in KG |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |  |
| PET | 1,18 | 1,18 | 1,69 | 3,93 | 1,35 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9,31 |
| Clear Plastic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0,00 |
| paper |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0,00 |
| garden |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0,00 |
| organic | 3,89 | 6,83 | 6,01 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 16,72 |
| metal | 5,07 | 6,43 | 1,11 | 6,51 | 0,74 | 0,46 |  |  |  |  |  |  |  |  |  |  |  |  |  | 20,31 |
| Hazardous waste | 0,41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0,41 |
| Textile |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0,00 |
| Cardbox | 1,4 | 1,015 | 1,44 | 1,44 | 1,72 | 1,64 | 1,375 | 1,4 | 2,035 | 1,385 | 2,01 | 1,585 | 1,985 | 1,525 | 2,805 | 2,19 | 2,15 | 2,15 | 2,695 | 33,95 |
| MCW | 2,155 | 6,79 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 8,95 |
| Nappies |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0,00 |
| HDPE | 2,345 | 1,355 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3,70 |
| Glass | 13,25 | 13,25 | 6,235 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 32,74 |
| Polysterene(foam pack) | 0,895 | 3,005 | 0,89 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 4,79 |
| other plastic | 1,535 | 2,265 | 4,37 | 2,07 | 2,125 | 1,305 | 1,56 | 0,26 |  |  |  |  |  |  |  |  |  |  |  | 15,49 |
| ewaste |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0,00 |
| Total |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 146,34 |

Waste classification pictures

Residual waste ready to be sent to New England Landfill site, Pietermarzburg.

NDZ Municipality has an annual contract with Enviro serve for collection and transportation of residual waste to New England Landfill site. The packaging waste at Himville Transfer Station is used by the cooperative for recycling purposes. However there is a lot of waste such as glass and cans stock piled due to lack of market. There is no compost activities that are taking place on site due to lack of chipping machine.

### Waste recycling, treatment and disposal

#### Status Quo of Waste Disposal Facilities

| Bulwer Waste Disposal Site |  |
| --- | --- |
| Status | Licensed |
| Total capacity | 6131 |
| Existing capacity | 0 |
| Available airspace | 6131 |

#### Status Quo of Waste Disposal Facilities

Bulwer Landfill Site

Bulwer landfill site is currently operational and is in possession of a valid licence. However the site does not meet the minimum landfill site standard as per National Environmental Management: Waste Act 59 of 2008.

Pictures

Creighton  Landfill Site

#### Status Quo of Waste Recyclers

The waste recycling activities are taking place at Himville Transer Station. This duty has been outsourced by the municipality and is performed by the SMME (Bazamile). However there is still a challenge of piled glass and cans within the Himville transfer station.

Although there are waste recyclers within the municipality there is still a need of more SMMEs that can implement recycling within Bulwer and Creighton landfill site.

#### Status Quo of other types of facilities

Himeville transfer station

### collection services

#### 2.5.1National Domestic Waste Collection Standards

| Item | Total number |
| --- | --- |
| Households | 29619 |
| Serviced households | 4456 |
| Unserviced households | 25163 |
| Indigent households | 0 |
| Unserviced indigent households | 0 |
| Service Level A: On-site appropriate and regularly supervised disposal | 0 |
| Service Level B: Community transfer to central collection point: | 0 |
| Service Level C: Organised transfer to central collection points and/or kerbside collection: | 0 |
| Service Level D: Mixture of Service Level B and Service Level C: | 0 |
| Total Serviced households as per the National Domestic Waste Collection Standards: | 0 |

National Domestic Waste Collection Standards Graph:

### Financing of Waste Management

#### Budget/Expenditure: Income and expenditure

| Item | Amount |
| --- | --- |
| Collection |  |
| Transportation | R 108000000000 |
| Capex-purchase (vehicles) | R -1001000000 |
| Maintenance | R 0 |
| Fuel | R 0 |
| Receptacles | R 0 |
| General | R 0 |
|  | R 0 |
| Subtotal | R 106999000000 |
| Governance |  |
| Staff (remuneration) | R 0 |
| Education and awareness | R 0 |
| IWMPS | R 0 |
| By-laws | R 0 |
|  | R 0 |
| Subtotal | R 0 |
| Disposal |  |
| Transfer station | R 0 |
| Disposal sites | R 0 |
| Acquisition of land, equipment | R 0 |
| Regulatory compliance, EIA’s and licence | R 0 |
|  | R 0 |
| Subtotal | R 0 |
| Total | R 106999000000 |

Budget/Expenditure graph:

#### Revenue sources

| Source | Amount |
| --- | --- |
| Funding sources | R 0 |
| MIG Funding | R 0 |
| Equitable share funding | R 0 |
| Revenue from waste disposal fees | R 0 |
|  | R 0 |
| Total | R 0 |

#### Organisational and institutional matters

Organogram:

## Desired end state

### Setting strategic goals, targets and indicators

| Goal 1: Promote recycling and recovery of waste |  |  |  |
| --- | --- | --- | --- |
| Objectives | Targets | Activities | Timeframe |
| Contribute towards the growth of the green economy | Encouraging the participation of SMEs and cooperatives in waste service delivery and recycling | - Extend street cleaning activities in un-serviced areas - provide land for companies to implement viable recycling projects - investigate viable green projects (composting/biogas) - Encourage and support SMMEs and cooperative in partnership with industries such as PETCO, MONDI, SAPPI, Hotels and Commercial Businesses | - 2019 - 2021 - 2021 - 2020 |
| Promote waste minimisation | - Introducing the sorting from the households, businesses, government, municipal buildings and soldier camps, hotels - Introducing waste minimisation projects | - Separation at the source projects(provide different receptacles in the office(different bins colours) - Support school recycling projects - Establishment of a buy back centre and drop off facility | - 2020-2023 - 2019- 2023 - 2022 |

| Goal 2: Ensure the effective and efficient delivery of waste services |  |  |  |
| --- | --- | --- | --- |
| Objectives | Targets | Activities | Timeframe |
| Ensure/Provide effective and efficient delivery of waste services to the community. | - Rendering of waste removal services to communities (households) - Upgrading of a Landfill Site - Conversion of a Creighton Landfill Site into a waste transfer station. - Upgrading of a Transfer Station | - Provide waste removal services in Bulwer, Donnybrook, Underberg, Creighton. - Provide receptacles for household waste and CBD - Clear illegal dumping (schedule quarterly clean-up campaigns) - Adopt open spaces for greening parks and creation. - Purchase of specialized vehicles. - Purchase adequate drums, skip bins and equipment for the removal of garden waste from residential areas. - Upgrading (expansion) of Bulwer Landfill site - Feasibility study for the closure of the Creighton Landfill Site - Closure and Rehabilitation - Construction of a Creighton transfer station | - Ongoing - 2020 - 2019-2023 - 2023 - 2024 - 2021 - 2022 2023 2023 2023 |

| Goal 3: Ensure that legislative tools are developed to deliver on the Waste Act and other applicable legislation |  |  |  |
| --- | --- | --- | --- |
| Objectives | Targets | Activities | Timeframe |
| Development and implementation of Integrated Waste Management Plan. | - Updated IWMP - Gazetted Municipal by-laws - Landfill site operation plan | - Monitor IWMP implementation - Review IWMP and by-laws. - Development of landfill site operational plan by external service provider. | 2020-2023 2020 2021 |

| Goal 4: Sound budgeting and financing of waste management services |  |  |  |
| --- | --- | --- | --- |
| Objectives | Targets | Activities | Timeframe |
| Ensure the availability of budget for waste management services. | Participate on municipal budget. External funding. | - Annual allocation of budget for waste management services. - Apply for funding at MIG - Apply for funding at DEA | - 2019 - 2020 - 2020 |
| To ensure that the residents and businesses are charges waste collection tariffs. | - Tariff modelling for waste management services | - Token system | - 2021 |

| Goal 5: Ensure the safe and proper disposal of waste |  |  |  |
| --- | --- | --- | --- |
| Objectives | Targets | Activities | Timeframe |
| To provide the public with safe and accessible waste disposal facilities. | Licensed waste disposal facilities (Landfill) | Ensure that the waste disposal sites meet the minimum required standards (Upgrade of Bulwer landfill site) | 2023 |
|  | Compile landfill site and transfer station operation plans | Ensure the success on operation plans implementation. | 2021 |

| Goal 6: Education and awareness |  |  |  |
| --- | --- | --- | --- |
| Objectives | Targets | Activities | Timeframe |
| Initiation and implementation of community education and awareness campaign. | - Annual programme of awareness campaign. - 1 clean-up and awareness campaign per month. | - Strengthen relationship and participation of all stakeholders. - Schedule clean-up and awareness campaign. | - 2019-2023 - 2019-2023 |

| Goal 7: Compliance and enforcement |  |  |  |
| --- | --- | --- | --- |
| Objectives | Targets | Activities | Timeframe |
| Maintain and provide compliant waste disposal sites according to waste management licence/ norms and standards | Compliance with permit/ norms and standards requirements for waste facilities | Auditing of the sites and Implement audit recommendations | 2020-2023 |

## Identify, evaluate and select alternatives

### Strategic goals, targets, timeframe, budget

| Goal 1: Promote recycling and recovery of waste |  |  |  |
| --- | --- | --- | --- |
| Objectives | Targets | Alternatives | Budget |

| Goal 2: Ensure the effective and efficient delivery of waste services |  |  |  |
| --- | --- | --- | --- |
| Objectives | Targets | Alternatives | Budget |

| Goal 3: Ensure that legislative tools are developed to deliver on the Waste Act and other applicable legislation |  |  |  |
| --- | --- | --- | --- |
| Objectives | Targets | Alternatives | Budget |

| Goal 4: Sound budgeting and financing of waste management services |  |  |  |
| --- | --- | --- | --- |
| Objectives | Targets | Alternatives | Budget |

| Goal 5: Ensure the safe and proper disposal of waste |  |  |  |
| --- | --- | --- | --- |
| Objectives | Targets | Alternatives | Budget |

| Goal 6: Education and awareness |  |  |  |
| --- | --- | --- | --- |
| Objectives | Targets | Alternatives | Budget |

| Goal 7: Compliance and enforcement |  |  |  |
| --- | --- | --- | --- |
| Objectives | Targets | Alternatives | Budget |

## Communication and Stakeholder Participation

### Consultation Process Summary

| Stakeholder | Issues raised/ Concerns | Municipality’s response | General comments |
| --- | --- | --- | --- |

## Implementation Instruments

### Partnerships

### Legislative instruments: Development and enforcement of by-laws

### Funding mechanisms

### Implementation Plan (Summary of an IWMP Planning Process)

| Situation Analysis | Desired end state (Goals) | Targets | Action | Y1 (2019) | Y2 (2020) | Y3 ( 2021) | Y4 (2022) | Y5 (2023) | Selected alternatives | (Implementation mechanisms) Resources |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Human Resource (HR) | Equipment (EQP) | Finance (HR+EQP) |
| Lack of waste separation at source | Implementation of separation at source | Urban Households | Provide waste removal services in Bulwer, Donnybrook, Underberg, Creighton | X | X | X | X | X |  | Existing Staff and additional staff EPWPs, CWPs, | 2 bags system Clear bags and Black bags | 500 000 |
|  |  | Soldier Camps |  | X | X | X | X | X |  |  | 2 bags system Clear bags and Black bags |  |
|  |  | CBD |  |  |  |  |  |  |  |  | 2 bags system Clear bags and Black bags |  |
|  |  | Hotels |  |  |  |  |  |  |  |  |  |  |
|  |  | Rural & farms | Supervised on-site disposal | X | X | X | X | X |  | Existing Staff and additional staff EPWPs, CWPs, |  |  |
| Three active waste facilities sites, viz. Underberg, Bulwer & Creighton. All licensed according to NEMWA amended | Improve and develop infrastructure to comply with requirements of the legislation and municipal needs | Ensure that all the waste sites facilities | Upgrading of transfer station and proper management | X | X | X | X | X |  | Existing staff and service providers | Weighbridge, Weighpads, Proper office with the package, chipping machine | R 9 000 000 |
|  |  | in the Municipality are licensed and comply with legislative requirements. | ansion) of Bulwer Landfill site Feasibility study for the closure of the Creighton Landfill Site Closure and Rehabilitation Construc | X |  |  |  |  |  |  |  |  |
|  |  |  | tion of a Creighton transfer station |  |  |  |  |  |  |  |  |  |

## Reporting on Monitoring

### Strategic issues

### Performance

### Public accountability

### Communication and public participation plan

### Financial plan

### Waste management implementation programme

# Annexure or References (Optional)